C 417

Lavalier Microphone

Architect’s and Engineer’s Specifications

C 417 PP (C 417 L):

Lavalier condenser microphone with omnidirectional

polar pattern, applications: clip-on microphone for lecturers

(“handsfree”), theatre, stage miking. Bandwidth

20–20,000 Hz, sensitivity at 1 kHz: 10 mV/Pa, electrical

impedance 200 ohms, dimensions: 7.5 mm (0.3 in.) Ø,

length: 15 mm (0.6 in.), cable length: 3 m (10 ft.) (1,5 m

(5 ft.)), XLR plug with phantom powering adapter (3-pin

mini XLR), phantom powering: 9–52 V, supply current

approx. 2 mA, matte black finish, net (without

cable)/shipping weight: 68/220g (2.5/8.1 oz.) (8/160 g

(0.3/5.9 oz.)). Including accessories: windscreen, fixing

material.
