CK 62-ULS

Capsule Module

Ultra Linear Series

Architect’s and Engineer’s Specifications

Omnidirectional microphone capsule for the ULS modularstudio

condenser microphone system from AKG.

All acoustical parameters shall be highly linear. The

capsule shall provide a consistent polar pattern

throughout a wide frequency range. The capsule shall

be temperature stable and highly reliable, with a gold

plated contact pin for connection to the C 480 B preamplifier.

When attached to the C 480 B power supply

module, the capsule shall perform to the following

specifications: Frequency range: 20–20,000 Hz; sensitivity

at 1 kHz: 40/20/6.3 mV/Pa selectable; maximum

SPL: 134/140/144 dB SPL for 0.5 % T.H.D.; equivalent

noise level: 13/11/17 dB-A. The case shall be no larger

than 21 mm (0.8 in.) in diameter and 23 mm (0.9 in.)

long inclusive of the contact pin. The maximum net

weight of the capsule shall be 25 g (0.9 oz.).
