CK 69-ULS

Shotgun Capsule

Ultra Linear Series

Architect’s and Engineer’s Specifications

Shotgun microphone capsule for the ULS modular studio

condenser microphone system from AKG. The capsule

shall comprise a two-part interference tube to provide

a choice of two different shotgun capsules in one.

The capsule shall be temperature stable and highly

reliable, with a gold plated contact pin for connection

to the C 480 B preamplifier. When attached to the

C 480 B power supply module, the capsule shall perform

to the following specifications: Frequency range:

20–18,000 Hz; sensitivity at 1 kHz: 54/27/8.5 mV/Pa

selectable; maximum SPL: 134/140/142 dB SPL for

0.5 % T.H.D.; equivalent noise level: 11/9/15 dB-A. The

case shall be no larger than 21 mm (0.8 in.) in diameter

and 176/317 mm (6.9/12.5 in.) long inclusive of the contact

pin. The maximum net weight of the microphone

system shall be 70 g (2.5 oz.).
