CK 80

Capsule Module

Discreet Acoustics

Architect’s and Engineer’s Specifications

The microphone shall be a screw-on hypercardioid

capsule that mates with the AKG Discreet Acoustics

Modular Series and shall have an interference tube section

for enhanced directivity at high frequencies. The

microphone shall accept phantom powering over the

universal range from 9 to 52 volts dc. The response

shall have a gentle rise from low to high frequencies

such that the total frequency response from

60–15,000 Hz rises uniformly over an 18 dB range. The

microphone shall have a sensitivity no less than 30

mV/Pa (–30 dBV), and the total harmonic distortion

(THD) at an operating acoustical level of 125 dB shall be

no greater than 1 %. The electrical impedance shall be

no greater than 600 ohms. Electrical mating contacts

for mounting the microphone shall be gold plated for

positive, trouble free operation over long periods.

Net/shipping weight: 15/208 g (0.5/7.3 oz.).
