PT 4000

Wireless System

WMS 400 UHF

Architect’s and Engineer’s Specifications

The wireless system shall operate over a 30 MHz UHF frequency

range in one of 6 bands from 680 to 863 MHz, with a

wide range of possible frequency settings. The wireless

microphone system shall incorporate factory-optimized sets

of both transmission and receiving frequency modules that

are programmed into both transmitters and receivers. Each

frequency set shall comprise a 30 MHz UHF band and offer

up to 1,200 discrete operating frequencies within that band,

with transmitter power not exceeding 50 milliwatts. Both

handheld and bodypack type trans-mitters shall be provided.

The bodypack transmitter shall accommodate both microphone

and line level inputs. The transmitters shall operate

with a companding system that is complemented by the

receiver, providing an A-weighted system dynamic range in

excess of 120 dB. The transmitters shall indicate input overload

via an LED indicator and on a backlit display battery lifetime

shall be displayed in hours as well as the transmitting frequency.

Handheld transmitters shall have a maximum diameter

of 39 mm and a length, including antenna, no greater that

239 mm. The wireless microphone system shall be the AKG

Acoustics Model WMS 4000.
